Lead Abbreviations and Useful Terms

| BLL | Blood Lead Level: the concentration of lead in a person’s blood, expressed as micrograms of lead per deciliter of blood (ug/dl) |
| --- | --- |
| EPA | U.S. Environmental Protection Agency |
| HUD | U.S. Department of Housing and Urban Development |
| LBP | Lead Based Paint. Defined by EPA as any coating containing 1.0 milligrams of lead per square centimeter (mg/cm2) or 0.5% by weight (5000ppm) |
| OSHA | Occupational Safety and Health Administration of the U.S. Department of Transportation |
| PEL | OSHA’s Permissible Exposure Limit: The maximum daily average level of lead in the air that a worker is permitted to breathe. The PEL is 50 micrograms per cubic meter of air (ug/m3) over an 8 hour period. OSHA also has an action level of 30 ug/m3, which triggers blood testing if exceeded. |
| RRP | The 2008 U.S. EPA Renovation, Repair and Painting Rule |
| XRF | X-ray fluorescence analysis is the nondestructive, rapid and relatively expensive method used by certified risk assessors and paint inspectors to determine the level of lead in a given painted surface. |

Weights and Measures

29 grams is approximately equal to 1 ounce

1 gram (g) = 1000 milligrams (mg) = 1,000,000 micrograms (ug)

1 meter (m)  is approximately 39.3 inches

1 cubic meter (m3) is about 35 cubic feet

2.54 centimeters  (cm) is approximately 1 inch

25 square centimeters (cm2) is about 4 square inches (2” x 2”)

A 2 inch hole is equal to 3.14 inches2